Audio Convert V1.1   MELP to Wave File Converter

This is a small utility program to convert MELP codec audio files created by WinDRM into wave files for play back on any wave player. The Audio Convert’s file name is audioconv_melp.exe.  Put this file in the same directory with the melp_dll.dll file. Normally, this would be the WinDRM directory.

To make a voice recording:

Check Save Audio under WinDRM>Setup>Codec.  The input file audio.mlp file will then be created in the WinDRM directory. To stop the recording, uncheck Save Audio.  WinDRM creates only one file name (audio.mlp). However, this file may be renamed, just retain the mlp extension.  Note: If this input file can not be found (wrong file name) when “Start” is pushed, the program will crash.

To convert the mlp file to wav:

Execute Audio Convert and select the display and sound card preference.  Each recording may also be saved with a different file name.  Just rename the “Out” file (audio.wav) and retain the extension (.wav). If not renamed, then the next recording will be appended to the last one.  Push “Start” to begin the conversion. Upon completion, the wave file will be created and saved. The recorded audio.mlp file can not be played back (retransmitted).

The audio.mlp recording will be played back as it is being converted.

(01-December-2006  KØPFX)