June ‘99

BreezeNET PRO.11 – Release Notes & Documentation

Preface

This document consists of:

- BreezeNET Pro.11 Version 4.4.1 Release Note – June ‘99.

- BreezeNET Pro.11 Version 4.310 Release Note – Feb ‘99.

- BreezeNET Pro.11 Version 4.211 Release Note – Sep ‘98.

- BreezeNET Pro.11 Version 4.204 Release Note – July ‘98.

- MB 11 - BreezeNET PRO.11 – Aug 98.

For any further details related to the information included in this document, please do not hesitate to contact:

BreezeCOM Technical Support at http://www.breezecom.com/ContactUs/brzcnt.htm

Release Note: BreezeNET PRO.11Version 4.4

General

BreezeCOM is pleased to announce the release of BreezeNET Version 4.4 . This version adds the following features to the BreezeNET product line: Authentication and power save. Better accommodation for the wide range of countries where BreezeNET is used (see Hopping Standard and DL unit). Please find below a description of these new features, and others that are specific for the BreezeNET units and the SA-PCR.

An updated BreezeVIEW management application is now available.

Authentication

Wired Equivalent Privacy (WEP) authentication algorithm protects authorized Wireless LAN users against eavesdropping and access by un-authorized users. When enabled in an AP-10, only a station adapter that has the privacy option implemented and an identical encryption key will be able to associate to the access point. Authentication has been implemented as defined in the 802.11 standard using RSA’s RC4 algorithm.

This feature is an orderable option. You can either order a BreezeNET unit (including SA-PCR) with the privacy option factory installed, or you can upgrade to add the Privacy feature to existing units, it is also possible to return the units to BreezeCOM for upgrading.

When ordering new units or upgrading existing ones please use the following catalog numbers for the Privacy option:

811104 – For AP-10, WB-10, SA-40 and SA-10

811105 – For SA-PCR

The authentication parameters are configurable using the monitor or a SNMP management station.

#### Power Save

Power Save mode is primarily intended for the SA-PCR, usually installed on laptops or handheld computers, in order to conserve battery energy. The Power Save mode allows the SA-PCR to go into a Doze state for a predefined period that will reduce the PCMCIA power consumption. The settings for this option are made per cell and per station. Power Save needs to be configured in both the AP-10 and the SA-PCR. Performance of the whole cell will be effected even if only the AP and one station are set to Power Save Mode.

Power Save Mode has been implemented as defined in the 802.11 standard. The power save in the AP also works with Symbol's station adapter.

Country Standards Support

The BreezeNET PRO.11 including the PC card can now be ordered for the following standards:

US, Europe, Japan, France, Canada, Russia (Site proprietary), Australia, Netherlands, Israel, Spain, Korea.

BreezeVIEW PC

An updated version of BreezeVIEW PC is released, which will work on version 4 or 5 of SNMPc. In this release support has been added for SNMP variables that were added in the BreezeNET PRO.11 product family. This version also supports BreezeNET PRO products.

SNMPc version 5 (workgroup edition) operates on Windows 98/NT only. To purchase SNMPc and for more information on this platform please refer to www.castlerock.com

BreezeNET PRO.11 (AP-10 SA-10/40 WB-10) Release 4.4 Details

Firmware = 4.4.1

MIB = 2.6a

Authentication, Power Save and Country Standard Support

# See details above

DL Model

The power level in a DL unit is set to Low Power, and cannot be changed to High Power. The power level is not reset by Set Factory Default at any level. This model is targeted for ETSI-compliant installations using high gain antennas.

The catalog numbers are as follows:  	811150P  -  AP-10DL PRO.11

811151P  -  SA-10DL PRO.11

811152P  -  SA-40DL PRO.11

811153P  -  WB-10DL PRO.11

VLAN Support

There is now support for VLAN (802.1Q). The support for VLAN is transparent support, that is VLAN traffic will be transferred over BreezeNET equipment without interference but will not actively take part in this protocol.

MIB

There is now a new version to the BreezeCOM MIB. There are new instances for the setting and configuration of Power Save mode and Authentication.

To upgrade to version 4.4.1, the upgrade package, is obtainable from Technical Support area on the BreezeCOM web site http://www.breezecom.com/TechSupport/techsupp2.html

Release Note: BreezeNET PRO.11 Version 4.3

General

Interoperability

BreezeNET PRO.11 products interoperability has been tested with other companies’ 802.11 WLAN products. Intensive testing has been successful in a mixed environment of BreezeCOM and Symbol products with their latest versions. Both with our stations working (SA-10, SA-40 and SA-PCR) with Symbol's access point and vice versa.  The testing with other companies’ products is in advanced stages.

Note: The test was done using Symbol’s products Version 4.0

Performance

Major improvements have been made to increase the performance of all of the BreezeNET PRO.11 product family. The improvement is most noticeable in specific situations, for example where there is high ratio of short packets and other situations.

ODI Driver for SA-PCR

An ODI driver for PC's is now being released after successful Beta testing. The ODI driver supports Novell VLM, NETX clients, Novell TCPIP, Novell Intranetware clients, Lantastic, and LAN Manager clients (NDIS2). This driver provides a solution for NDIS2 environment, using the driver ODINSUP above our ODI driver. Thereby providing a NDIS2 interface to the NOS (Network Operating System).

Note: The ODI drivers for handhelds are now available for Beta. See Pending Release Document

BreezeVIEW PC

An updated version of BreezeVIEW PC is released. In this release support has been added for SNMP variables that were added in the BreezeNET PRO.11 product family.

This version also supports BreezeNET PRO products, but if you try and get the value of a variable that was not available in the BreezeNET PRO a value of UNK will be returned.

BreezeNET PRO.11 (AP-10 SA-10/40 WB-10)

Firmware = 4.310

New Features and Improvements:

- Monitor

The monitor has been reorganized. The new layout is now user friendly and easier to use. The site survey menu now contains all the statistic counters, site survey software, event log and more.

- Site Survey Software

The Survey software has been modified. All configuration information is now

taken from the WLAN configuration, such as antenna, power level, etc.

- Bug Fixes

Download

Sometimes a failure occurred during the download process. This has been corrected.

Roaming

Seamless roaming of SA-40 did not work in hierarchical installations. The SA-40 now performs high speed seamless roaming.

Long Range

There was a problem that caused retransmissions and duplicates in very long links. This problem has been addressed and can operate on extremely long links. Long range should be enabled only when the link is over 20km. This option can also be configured using a SNMP Management station.

To upgrade to version 4.3 , the upgrade package, is obtainable from Technical Support area on the BreezeCOM web site www.breezecom.com.

Release Note: BreezeNET PRO.11 Version 4.211

# Changes

Reason: This version is being released due to minor hardware changes.

Effected Models: All BreezeNET PRO.11 models.

Effective Date: 9 September 1998

Result: This does not have any effect on functionality or performance of the 
             products at all.

## Note

- Added MIB variable, brzACKDelayed, to support long range. This variable is equivalent of the “Long Range On/Off” option in the monitor.

- Load sharing is not backward compatible. Meaning that the load sharing feature does not function in versions prior to 4.211, all other functions are fully compatible.

- If you have a management station don’t forget to recompile the new mib file for PRO.11.

# Upgrading

To upgrade BreezeNET to version 4.211 please refer to the appropriate package, see details below.

To upgrade from:

- BreezeNET PRO V3.50 and earlier versions use file “Upgrade_350_to_4211.exe”

- BreezeNET PRO V3.52 and BreezeNET PRO.11 V4.204 use file

“Upgrade_352_to_4211.exe”

- BreezeNET PRO V3.62 and BreezeNET PRO.11 V4.210 use file

“Upgrade_362_to_4211.exe”

For future upgrading of BreezeNET PRO.11 V4.210 and later versions the filenames have been changed.

|  | New File Name | Old File Name |
| --- | --- | --- |
| Erase Flash | erase_fw | Erase |
| Firmware for SA or WB | sawb_fw | Eansfb |
| Firmware for AP | ap_fw | eanafb |

Release Note: BreezeNET PRO.11 Version 4.204

Introduction

The scope of this document is to point out the noticeable changes after upgrading a BreezeNET PRO unit with the PRO.11 firmware. This document does not describe the new features and benefits of the PRO.11 software. It is intended to give people who are familiar with the BreezeNET PRO units an overview of changes in parameters and configuration options, which existed in the previous versions. It will also give system administrators some installation and configuration tips for upgrading an existing installation.

If you want to learn about the many new features in this version, please refer to the user manual.

- Upgrade Procedure

The firmware upgrade procedure should be done according to the firmware upgrade application note.

- BreezeNET PRO units from versions up to 3.50, including 3.50 should be upgraded with the files: download, eanaf and eansf.

- BreezeNET PRO units from version 3.52 or higher should be upgraded with the files: erase, eanafb and eansfb.

- Wireless firmware upgrade

The BreezeNET PRO version 3.52 was the first to allow wireless firmware upgrade for remote station adapters. Please note that when upgrading an installation wirelessly, the most remote stations, by means of topology, should be upgraded first.

Example: In the drawing below, You should first upgrade the SA-10, then AP-10 (b) then the WB-10 and last AP-10 (a).

TFTP Host

AP-10

(A)

## WB-10

## SA-10

AP-10

(B)

Step 4

Step 3

Step 2

Step 1

Fig. 1: Wireless Firmware Download Order

- Backward Incompatibility

Please note that since the BreezeNET PRO.11 complies with the final 802.11 standard, it is not compatible with previous versions of BreezeNET (i.e. any 3.xx version). The implication of that is that you cannot have 3.xx station adapters connecting to PRO.11 and vice versa.

Another implication is that if you have co-located cells of different versions, you might see warning messages on the monitor port, of the units hearing  “incompatible” frames. However, no functional failure is expected because of that.

- MAC addresses change

The 802.11 version includes a change in the structure of the units Ethernet addresses, as a consequence, the MAC address of a unit will change after this firmware upgrade. Obviously it won’t change to a MAC address that already exists, so there isn’t any problem with that. Notice that the MAC address of the unit is used only for its firmware upgrade or for SNMP management of it, so it does demand you to refresh the ARP tables of your SNMP manager (the easiest way is just to reset it).

- Hopping sets and patterns

The system of selecting a hopping sequence is now changed. The 802.11 standard defines three hopping sets, each includes several hopping patterns (26 for US FCC and Europe ETSI). A hopping pattern is the name the 802.11 standard gave to a hoping sequence. The standard defines the patterns in 3 different sets in a way that whenever you choose different patterns from the same set, they have the best chances not to collide. The implication of that is that when installing two or more access points in the same area (like in a roaming environment or when a high bandwidth solution is needed), different hopping patterns from the same hopping set should be selected.

Under the Wireless LAN Parameters menu there is now a new option – Hopping Set. When configuring a hopping set, you will be able to select only hopping patterns that belong to this set. The default configuration is hopping set 2, hopping pattern1.

The patterns are divided in the following way (US & Europe) :

Hopping Set 1: 0,3,6,9,…,75

Hopping Set 2: 1,4,7,10,…,76

Hopping Set 3: 2,5,8,11,…,77

After upgrading a multi-cell environment, you should make sure that all of the AP’s are configured with Hopping Patterns within the same set.

6. IP Subnet Mask

Unlike the current BreezeNET PRO versions, in the BreezeNET PRO.11 units, when you configure a unit with an IP address, you must manually enter the IP subnet also. The unit will not set the subnet to the default compatible IP subnet. The reason for that is to give more freedom in selecting a subnet, and to match the usual situation when putting an IP address to any device.

When downloading the new firmware, it will use the old value of the subnet, so if your subnet mask was set correctly it will remain so.

- Bridging policy is changed

The PRO.11 includes a change in the existing bridging modes and also a new bridging mode and a new aging mechanism to improve the aging process.

Reject Unknown – The AP will only learn addresses from the wireless side, and it will transmit to the wireless media only frames with a destination address of a wireless station (either connected directly with a SA-10, or a station that is connected to a SA-40 or a WB-10)  that has already transmitted before.

Forward Unknown – The bridging policy was improved for this mode, and now it works in the following way: The AP will learn the addresses on both sides. If it receives a frame from the Ethernet side where the destination address is unknown, it will transmit it to all the wireless stations. As soon as the AP learns that a station is on its Ethernet side, it will not transmit any frames addressed to this station to the wireless side.

Intelligent Bridging – A new bridging mode, suitable for an environment with roaming WB-10’s .  The AP will work in the Reject Unknown mode. As soon as a WB-10 will roam in, the AP will go to a special mode where it will maintain seamless connectivity with all the stations connected to the WB, and learn their addresses. The AP will stay in this mode to a configurable period, when at the end of this  period it will go back to the Reject Unknown mode (assuming that in this time it learned all the wireless addresses).

- AP Redundancy Support

In the PRO.11 version, the time for the “no Ethernet” decision can be configured by either monitor or SNMP management.

Via the monitor you can enter a value between 0 to 255, where 0 means disabling the feature and the other values represent number of seconds for the decision.

Via the SNMP management there are two MIB variables under SysParams:

ApRedundancySupport – Enable/Disable the redundancy feature.

ApRedundancyLimit – Time in seconds for decision.

This feature works in the following way: If you configure the time for decision to X seconds, then every X seconds the AP will check if there was no Ethernet traffic during the last X seconds, and if not it will stop sending beacons , causing the stations associated to it to roam to another AP. The implication of that is that the maximum time for the AP to stop transmitting beacons from the moment the Ethernet traffic stopped is two times X.

- Support for SA-40 Fast Roaming

The PRO.11 version includes support for SA-40 seamless roaming. When a SA-40  roams into an AP-10, the AP will send to the Ethernet side four frames with the source addresses of the stations connected to the UTP ports of the SA-40. Those frames enable an update to the network topology, so the location of those stations will be known to all the various network components.

- New MIB file

A new MIB file is released. This MIB file should be compiled in your SNMP management station to enable management of the BreezeNET PRO.11 units.

- New Standards

Support for new hopping standards were added and the Japanese and the French hopping standards are now different in order to be compatible with the 802.11 final definitions. If you are upgrading BreezeNET units in France, Japan, Korea, Spain or The Netherlands please contact BreezeCOM for instructions on how to set the units to the new hopping standard.

Marketing Bulletin   # 11

Contents:  	1.- Release of BreezeNET PRO.11 Series (ver.4.204) - IEEE 802.11

compliant

2.- Release of SA-PCR and SA-PCD - IEEE 802.11 compliant

- Firmware ver 4.1.13

- Appl ver 4.1.1

- Driver for Win95 - ver 4.1.1

3.- Release of Enhanced Management Software Applications

4.- Multi-vendor Interoperability Issues

5.- PRO.11 Series to PRO Series Compatibility Issues

6.- Upgrading Policies

1.- Release of BreezeNET PRO.11 Series - IEEE 802.11 compliant

We are pleased to announce BreezeCOM’s new BreezeNET PRO.11 Wireless LAN Series.

A leader in the wireless LAN market, BreezeCOM is the first vendor to have a complete family of IEEE 802.11 compliant products.

The new PRO.11 Series includes:

- AP10 / AP10D PRO.11 - Wireless Access Point

- SA-PCR / SA-PCD PRO.11 - Wireless PC Card Adapters

- SA-10 / SA-10D PRO.11 - Single Port Wireless Station Adapter

- SA-40 / SA-40D PRO.11 - Four Port Wireless Station Adapter

- WB-10 / WB-10D PRO.11 - Wireless Ethernet Bridge

For the BreezeNET AP, SA and WB, the new series is just a S/W upgrade from the present PRO Series. The new S/W version is 4.2.

The SA-PCR and SA-PCD are entirely new cards with new firmware and drivers.

1.1.- New features

1.1.1.- IEEE 802.11 compliance

a.- New and modified hopping standards and hopping sequences (in full accordance with the 802.11 standard) to support the following countries: France, Japan, Korea, Spain, and Holland. The hopping sequences are now arranged in hopping sets.

b.- Many other modifications and additions, without influence on the units’ functionality, but extremely relevant for interoperability with other vendors’ products.

“Shared Key authentication” and “power saving” are currently being tested and will be released shortly.

1.1.2.- Other new features

a.- Increased throughput - More than 2 Mbps throughput for LAN data (2Mbps for FTP).

b.- Translation bridging - BreezeNET PRO.11 Series support translation / transparent bridging as defined in IEEE 802.1.h and RFC 1042.

BreezeCOM took the initiative to define the bridging algorithms, based on the above-mentioned standards, and it seems that most of the vendors participating in interoperability tests with us have agreed to use them. Encapsulation mode is supported for interoperability with vendors who have not yet released translation mode.

c.- Improved load sharing - Where multiple APs are available, BreezeCOM stations are able to select the AP with the lowest load, and therefore the load distribution among collocated APs will be quite equal. Even if the network has a mix of BreezeCOM and non-BreezeCOM stations, our stations will be able to move to the APs with lower load, improving the overall performance of the network.

d.- Configurable WLAN to WLAN relaying function of the AP - In some installations the capability of one station to transmit to another station within the same cell via the AP may need to be disabled. In the AP-10 PRO.11, this may be done via a management command.

As activating the option will prevent users from communicating with each other, it is recommended that this option be activated only in pure client/server environments, when client to client communications are never required.

e.- Improved AP redundancy - In collocated cell environments, upon failure of an AP, its stations will switch to other available APs. The period of time it takes can be configured between 250ms to 1sec via the monitor or SNMP.

f.- Enhanced performance in large networks – An improved bridging and aging mechanism allows the AP-10 PRO.11 and WB-10 PRO.11 to handle larger networks more efficiently.

g.- Improved performance in noisy environments – A configurable noise level threshold allows BreezeNET PRO.11 units to transmit despite noisy conditions.

h.- More options for AP selection – A BreezeNET PRO.11 station will attempt to associate with the best AP available. If a “good” AP is not available, another, less capable AP will be selected nonetheless. Additionaly, a configurable option forces a BreezeNET PRO.11 station to always associate with a specific ("preferred") AP.

i.- Seamless roaming for the SA-40 - The SA-40 PRO.11 is now able to roam seamlessly, even if the attached PCs are not able to transmit after roaming into a new cell.

j.- Enhanced roaming for the WB-10 - Even if stations connected to the WB-10 PRO.11 do not transmit anything after roaming into a new cell, the AP they roam to will forward frames to them. After the first transmissions are made by the stations, the AP learns of their presence, this feature is called "intelligent bridging".

1.2.- General

- A new MIB, ver.2.2 was released, to support BreezNET PRO.11, ver.4.2

- New documentation was released:

- Data sheet - cat. no. 213023, rev.0

- Installation Guide - cat. no. 213025

- Reference Guide - cat. no. - 213026

- Quick Installation Guides

- AP-10 PRO.11 – 213028

- SA-PCR PRO.11 - 213027

- SA-10 PRO.11 - 213029

- SA-40 PRO.11 - 213030

- WB-10 PRO.11 - 213031

2.- Release of SA-PCR and SA-PCD - IEEE 802.11 compliant PC Cards

Features

a.- 802.11 Compliant - Fully compliant with the IEEE 802.11 standard.

b.- Form Factor - Extremely compact with dual antennas (diversity). The SA-PCR has two built-in retractable antennas. The SA-PCD has two connectors to which detachable antennas may be connected.

c.- Rate Over the Air - 1, 2, and 3 Mbps with multi-rate support for efficient use of the medium (as a function of distance and / or quality of link).

d.- Throughput - Up to 2 Mbps of raw data. The highest in the market !

e.- Operating Systems Supported – Windows 95. In the near future ODI, Windows NT and Windows 98.

f.- Load balancing - Supported, based on BreezeCOM’s proprietary algorithm also used in stand alone units and the AP-10 PRO.11.

g.- Fast Seamless Roaming - based on proprietary algorithm.

h.- Site Survey Application - A very useful and user friendly Site Survey application is available. It allows to check and record the signals received by the PC card in different locations, providing a clear image of available coverage. Data gathered in the field is later analyzed and decisions to add or remove APs are easily taken.

i.- Configuration and setup - Performed via the PC; the parameters are saved on the local disk. Configuration files may be imported (for fast installation) and exported (for installation on different machines or for diagnostic purposes).

j.- LED Display - Association and traffic LEDs are available, indicating different operating states.

3.- Enhanced Management Software Applications

An extended MIB supports collecting and viewing of statistics per station. Received frames per frequency may be seen for remote site survey applications and additional WLAN condition traps contribute to robust connectivity. We are pleased to release BreezeVIEW OV for beta testing. This application is an enhanced graphic application for monitoring and managing the BreezeNET PRO.11 family of products and the wireless LAN environment. This application runs on the HP OpenView for Sun network management platform.

BreezeVIEW PC release 2 has been enhanced to support the BreezeNET PRO.11 family of products and is also being released for beta testing.

4.- Multi-vendor Interoperability Issues

Please note that compliance of two products to the same LAN standard (such as IEEE 802.11) does not imply that the users of the said products will be able to communicate.

This is because the communication process between users has many aspects, and LAN standards define only part of them.

Think about sending a letter to a friend. For your friend to be able to “receive” the letter, you should agree on using a specific language on the envelope (for him to be able to understand that the letter is addressed to him) and you should also agree on the language to be used in the letter itself.

The problem with LAN standards (including IEEE 802.11) is that they define only the envelope, and they do not care about the letter itself. Other problematic aspects include translation / transparent / encapsulation and bridging.

Conclusion: Products from different vendors may be declared as being compatible, only if they have been checked and found to be compatible.

We are in the process of checking our PRO.11 Series against other vendors (Symbol, Raytheon, NetWave/Bay and Aeronet) as part of the tests made in the Inter Operability Laboratory (IOL) of the University of New Hampshire (UNH). Before committing, please check with your sales contact at BreezeCOM for the latest news on interoperability.

5.- PRO.11 Series and PRO Series Compatibility Issues

PRO.11 Series and PRO Series products do not communicate. The AP and its stations should always be of same family (either PRO.11 or PRO). While BreezeNET PRO based cells may coexist with BreezeNET PRO.11 cells on the same Ethernet LAN, if cells are wirelessly overlapping, it is recommended that all units be of the same family.

6.- Upgrading and Upgrade Policy / Pricing

6.1.- 	Stand alone units - for stand alone units, PRO.11 is simply a new S/W version (version 4.2). An upgrade is performed as before, by downloading the new S/W.

a.- PRO versions up to and including 3.50, have to be provided with the following files:

eanaf, eansf, download

b.- PRO version 3.52 has to be provided with the following files:

eanafb, eansfb, erase

c.- Pricing

- downloading the new version from BreezeCOM’ web site: free of charge

- receiving the upgrade files on magnetic media: 25 USD transfer price

- Upgrade performed by BreezeCOM, including H/W upgrade to the latest H/W version: contact your sales person at BreezeCOM for part numbers and pricing

6.2.- PC Cards

Trade in old SA-PC PRO cards for the new SA-PCR / SA-PCD PRO.11. For every SA-PC PRO returned to BreezeCOM in exchange for a new SA-PCR / SA-PCD PRO.11, 100 USD will be deducted from the transfer price of the PRO.11 card.

6.3.- BreezeNET version 2.71 will be discontinued, effective January 1, 1999. BreezeNET version 3.52 (BreezeNET PRO) will continue to be sold alongside version 4.2 (BreezeNET PRO.11).