Application Note: Software Download Procedure                                              June. 99

Valid For:   BreezeNET PRO Series  Version 3.62

BreezeNET PRO.11 Series Version 4.210 / 4.211 and version 4.3.10

This application note describes the procedure of upgrading a unit to PRO.11 ver. 4.4.1 from a unit which is currently working with PRO ver. 3.62 or PRO.11 ver. 4.210/4.211 or ver. 4.3.10

Requirements:

1. PC or workstation with an Ethernet NIC and RJ-45 connection, and terminal emulation. A typical configuration is a PC running Windows with one MS-DOS window and one Hyper Terminal window.

2.  Monitor cable (Supplied with AP-10 unit)

3.  10-Base-T cable:

- Regular cable for download of SA-10 or SA-40

- Cross-over cable for download of AP-10 or WB-10

- Self-extracting 'upgrade 4_310 to 4_4_1.exe' file.

- Run/execute   'upgrade 4_310 to 4_4_1.exe'.

The directory should contain the following files after execution:

a. 3_62inst.txt - Download instructions in text format.

b. 3_62inst.doc - Download instructions in Word format.

c. Release notes BreezeNET Pro.11

d. Tftp.exe - tftp executable.

e. erase_fw - null file used during download.

f. Sawb_fw - firmware for station adapters (SA-10, SA-40, and WB-10).

g. Ap_fw - firmware for access point (AP-10).

h. Brz11prv.mib -MIB file for SNMP management.

Downloading procedure:

1.  Connect the unit to the power supply.

- Connect the unit to the terminal emulator using the monitor cable and start a monitor session - see System

Administrator guide section 3 for details.

3.  Set the unit's IP address (as described in the Installation guide section 3).

4.  Verify that the PC's IP address belongs to the same subnet.

5. Connect an Ethernet cable between the unit and the PC.

6. Use a regular cable when downloading a Station Adapter and a crossed cable when downloading an

Access Point or Wireless Bridge.

If the PC Ethernet adapter has a LINK indication, it should be on at this point.

7. PING the unit's IP address. Make sure that PING replies are being received.

The first one or two PINGs may fail, the rest should be received correctly.

It is recommended to open up the monitor window as well as the dos window in order to see the messages coming up on the screen in the downloading process.

8. Change directory to where files have been extracted. The default setting is c:\BreezeCOM\Versions\upgrade 4_310 to 4_4_1\. Copy the relevant firmware’s and the tftp application to a temporary folder (with a name of less than 8 characters) and run the downloading commands from there.

9. Use the tftp command to download the null file 'erase_fw' using the following

syntax:

tftp -i hostaddress put sourcefile [destinationfile]

-i  for binary mode.

Hostaddress is the Ip address you gave the unit in step 4.

Sourcefile is filename to be read from the current directory.

Destinationfile is filename to be written to unit.

Example: c:\temp\>tftp -i 206.25.63.55 put erase_fw erase_fw

10.  The unit's monitor should show the message:

"Erasing flash. Wait until completed before download."

11.  Wait a few seconds until the message:

"Flash erased, ready for download"  appears on the monitor.

DO NOT PROCEED UNTIL THIS MESSAGE APPEARS.

12.  Use tftp to put software on unit

PUT SAWB_FW  for SA-10, SA-40 or WB-10

PUT AP_FW  for AP-10

Example: c:\temp\>tftp -i 206.25.63.55 put sawb_fw sawb_fw(for SA-10, SA-40,  WB-10).

Example: c:\temp\>tftp -i 206.25.63.55 put ap_fw ap_fw (for AP-10)

This will take a few minutes.

- When software download is completed the unit will reset itself. After the unit recycles, check the monitor

message and make sure the version number (shown at the top of the main menu) is correct.

DO NOT  DISCONNECT ANY CABLES OR TRY TO STOP THE PROCESS BEFORE

DOWNLOADING IS COMPLETED. ATTEMPTING TO DO SO WILL DESTROY THE FLASH MEMORY. IN CASE OF POWER SUPPLY DOWN DURING THE TFTP SESSION THE FLASH MEMORY WILL BE DESTROYED.

IF FOR ANY REASON THE TFTP SESSION FAILED, STOP THE TFTP SESSION AND START AGAIN THE DOWNLOADING (FROM STEP NO. 7) WITHOUT RESETING THE UNIT OR DISCONNECTING IT.